Architecture that I could see based on implementation

Docker

Swagger Browser/Postman(REST API)

PostgreSQL

Technician

Simulator

Robot Client .Net Console

Server

Port:7777

Http:7779

ASP.NET

Dapper/SQL

P

gRPC:7778

- Port 7777 : PostgreSQL Database

- Port 7778 : gRPC (MagicOnion) for robot connections

- Port 7779: Http Rest API for front end/Technician

Modified CommandController

Prevent manual command

gRPC via IconnectedRobotCollection

REST

Robot

Pre robot state machine

AutoPilotSession

SortingSet/state/statistics/cache

AutoPilotManager

ConcurrentDictionary<string,autopilotsession>

Singleton

AutoPilotController

activate/deactivate/state/statistcs

Front End UI

Responsibilites:

AutoPilotController - With Scoped DI, Its per request - Exposes activate/deactivate/state/statistics endpoints to be operated by Front end team.

AutoPilotManager - With Singleton DI, Manages sessions per robot using ConcurrentDictionary<string, AutoPilotSession>.

Manages coordination between controller, gRPC hub and each session.

Making use of IServiceScopeFactory to reolve the DI scoped ISentCommandRepository while persisting commands.

AutoPilotSession - Works as State Machine, Tracks the

State : Deactivated or Running Or Paused

SortingSets :  ScanEnvironment/MoveToUnsortedBin/FaceUnsortedBin/ScanUnsortedBin/PickItem/

MoveToLabeledBin/FaceLabeledBin/ScanLabeledBin/PlaceItem

Statistics :

RobotHub  -

Works Per connection basis

On any of the event command result received + advance sorting - notify to AutoPilotManager,

Flage reported : pause,flag cleared, resume

CommandController - With Scoped DI, checks if autopilot is running then returns 409 Conflict -> This prevents from manual commands interference.